Dosing Skid Systems

LINESIG LNS-DZ Series

Reliable Dosing. Precise Control. Multi-Industry Flexibility.

The LINESIG LNS-DZ Series is a modular family of skid-mounted dosing systems engineered for controlled chemical feed, additive injection, inline blending and process metering across terminals, refineries, chemical plants, utilities and general industry.

The family is based on four pump technologies—centrifugal, diaphragm, piston/plunger and precision micro gear—integrated with application-matched flow measurement, pressure protection, process instrumentation, local control and PLC/DCS/SCADA interfaces.

Rather than applying one pump principle to every service, the LNS-DZ Series uses an application-specific engineering approach for:

medium- and high-flow dosing or blending,

corrosive and hazardous chemical metering,

high-pressure chemical or hydrocarbon injection,

very low-flow precision additive and catalyst dosing.

This allows pump technology, materials, pressure class, filtration, pulsation control, flow measurement and automation to be selected according to the actual liquid and operating conditions.

LNS-DZ PRODUCT FAMILY

| Model | Pump Technology | Primary Duty | Primary Engineering Focus |
| --- | --- | --- | --- |
| LNS-DZ-CEN | Centrifugal | Medium/high-flow dosing and blending | Smooth flow, hydraulic selection, multi-line control |
| LNS-DZ-PDD | Diaphragm metering | Corrosive/hazardous chemical dosing | Chemical compatibility, calibration, leak isolation |
| LNS-DZ-PDP | Piston / plunger | High-pressure process injection | Pressure protection, pulsation, packing design |
| LNS-DZ-PDG | Precision micro gear | Very low-flow additive dosing | Fine filtration, viscosity control, repeatability |

Common LNS-DZ Advantages

Skid-mounted, factory-integrated construction

Four application-matched pump technologies

Single, duty/standby and multi-line configurations

Independent dosing lines where required

Flow, pressure and level instrumentation

Ratio, batch and closed-loop flow control

Local, PLC, DCS and SCADA operation

Project-specific materials and pressure classes

Calibration and flow-verification options

Pressure relief and back-pressure protection

Pulsation control for reciprocating pumps

Suction filtration selected to pump technology

Injection check valve and quill arrangements

Duty/standby automatic changeover

Emergency shutdown and process-permissive interfaces

Hazardous-area electrical and instrument options

Modular and customizable design

Reduced field piping and cabling

Factory assembly and pre-testing

Indoor and outdoor construction

Drip tray, drainage and leak-detection options

Engineering documentation and test records

Custom Engineering Options

The LNS-DZ family can be configured with:

local day tank or remote suction source,

single or common suction manifolds,

single, duplex or fine filtration,

calibration columns,

suction and discharge pulsation dampeners,

mechanical pressure-relief valves,

back-pressure valves,

injection valves and quills,

Coriolis, positive-displacement, magnetic or suitable process flowmeters,

motorized or pneumatic control valves,

VFD or servo speed control,

electric stroke-position control,

ratio and batch dosing logic,

automatic duty/standby changeover,

heat tracing, insulation or temperature conditioning,

sampling, flushing, vent and drain connections,

PLC/HMI, remote I/O and communication interfaces,

coated carbon-steel or stainless-steel skid frames,

custom hazardous-area equipment selection.

LNS-DZ Family Comparison

| Engineering Feature | LNS-DZ-CEN | LNS-DZ-PDD | LNS-DZ-PDP | LNS-DZ-PDG |
| --- | --- | --- | --- | --- |
| Flow tendency | Medium to high | Low to medium | Low to medium | Very low to low |
| Pressure capability | Low to moderate | Moderate to high | High to very high | Low to moderate |
| Flow character | Continuous / smooth | Pulsating | Pulsating | Near-continuous |
| Corrosive liquid suitability | Conditional by MOC | Excellent | Good with metallurgy | Conditional by MOC |
| Very low-flow accuracy | Limited | Very good | Very good | Excellent |
| High-pressure injection | Limited | Good | Excellent | Limited |
| High-flow blending | Excellent | Not primary | Not primary | Not primary |
| Cleanliness sensitivity | Moderate | Moderate | Moderate | High |
| Primary capacity control | FCV / VFD | Stroke / speed | Stroke / speed | Precision speed |
| Pulsation control | Normally not required | Often required | Usually required | Normally not required |
| Typical skid duty | Blending / transfer | Chemical metering | High-pressure injection | Fine additive dosing |
| Main design driver | Hydraulics / flow | Compatibility / accuracy | Pressure / pulsation | Cleanliness / viscosity |

Typical Control:

| Dosing Control | Package Protection | Plant Automation Integration |
| --- | --- | --- |
| Local or remote setpoint Fixed-flow control Ratio to main process flow Preset batch quantity Flow totalisation Pump speed or stroke command FCV control where used | Low source level inhibit Low/no-flow alarm High discharge-pressure trip Mechanical pressure relief Pump/VFD fault Leak alarm where provided Local and remote ESD | PLC DCS SCADA Terminal automation Batch management Historian Remote I/O |
| Live flow and pressure display Duty/standby selection Automatic changeover Line permissive status Alarm history Operator access | Check/injection valve Back-pressure control Filter DP alarm where used Earthing/bonding interface Drip tray or drain monitoring | Modbus RTU Modbus TCP Industrial Ethernet Analog I/O Digital I/O Pulse / frequency |

Typical LNS-DZ Process Architecture

Chemical Source / Day Tank → Suction Isolation → Strainer / Filter → Selected Dosing Pump → Pulsation Control where applicable → Pressure Protection → Flow Verification / Metering → Control or Injection Valve → Check Valve / Injection Quill → Process Line, Tank, Manifold or Gantry Header

The exact process sequence is modified according to service.

For example:

centrifugal systems may use flow-control valves and/or VFD control;

diaphragm systems may include calibration columns and back-pressure valves;

piston/plunger systems require engineered pressure protection and pulsation control;

micro-gear systems require clean liquid and fine suction filtration.

Model Selection

Choose:

LNS-DZ-CEN for medium/high-flow dosing, inline blending, loading-gantry additives and controlled liquid transfer.

LNS-DZ-PDD for corrosive, hazardous or leak-sensitive chemical metering.

LNS-DZ-PDP for high-pressure pipeline, refinery and process injection.

LNS-DZ-PDG for very low-flow catalysts, dyes, markers and specialty additives.

1. LNS-DZ-CEN

Centrifugal Dosing Skid System

Product Overview

The LINESIG LNS-DZ-CEN is a skid-mounted centrifugal dosing system developed for medium- to high-flow chemical feed, additive injection, inline blending and controlled liquid transfer.

It is designed for continuous-duty applications where smooth, non-pulsating delivery is preferred. The skid can serve one process line or multiple independent dosing points from a common suction source.

The package combines centrifugal pumps, flow measurement, pressure monitoring, control valves and/or VFD control in a modular arrangement.

Typical Media

Ethanol and oxygenates

Methanol

Fuel additives

Dyes and markers

Lubricity improvers

Water-treatment chemicals

Cooling-water and boiler-water chemicals

Selected solvents and process liquids

General industrial liquid additives

Key Features

End-suction, close-coupled or process centrifugal pumps

Smooth, continuous flow

Single, duty/standby or multi-working-pump configurations

Common or independent suction manifolds

Independent flowmeter for each dosing line

Motorized or pneumatic flow-control valve

VFD capacity control option

Pressure gauge and transmitter

Discharge check valve

Injection valve or quill

PLC/HMI or local junction box

Remote PLC/DCS/SCADA interface

Hazardous-area equipment where required

Engineering Approach

The LNS-DZ-CEN hydraulic design focuses on maintaining:

adequate NPSH margin,

stable operation near the selected pump operating range,

low suction and discharge losses,

accurate independent flow control,

reliable check-valve and injection performance,

acceptable viscosity and vapor-pressure conditions,

controlled response during ratio or batch dosing.

Final pump selection is based on the minimum, normal and maximum flow, differential pressure, static head, piping losses, fluid density, viscosity, vapor pressure, temperature and available suction conditions.

Advantages

Suitable for medium- and high-flow dosing

Smooth delivery with minimal pulsation

Efficient multi-line blending architecture

Independent flow measurement and control

Flexible single or common-standby arrangements

Good fit for fuel-terminal and gantry systems

Compact skid-mounted construction

Reduced field installation work

Easy integration with plant automation

Typical Applications

Fuel terminals

Loading gantries

Refineries

Tank farms

Petrochemical plants

Water-treatment plants

Power and utility systems

Industrial chemical plants

Inline blending systems

2. LNS-DZ-PDD

Diaphragm Metering Pump Dosing Skid

Product Overview

The LINESIG LNS-DZ-PDD is an engineered diaphragm metering skid for accurate chemical feed where corrosion resistance, leakage control and repeatable positive-displacement dosing are important.

The diaphragm separates the process liquid from the mechanical drive. This makes the system suitable for treatment chemicals, aggressive liquids, hazardous products and selected hydrocarbon additives.

The package integrates calibration, pulsation control, back-pressure regulation, mechanical relief, pressure monitoring and local or remote automation.

Typical Chemicals

Acids and alkalis

Sodium hypochlorite

Antiscalants

Coagulants and flocculants

Corrosion inhibitors

Biocides

Neutralizing chemicals

Methanol and selected additives

Boiler-water and cooling-water chemicals

Specialty reagents

Key Features

Mechanically or hydraulically actuated diaphragm pumps

Single, 1W+1S, 2W+1S or multi-line configuration

Calibration column

Suction and/or discharge pulsation dampener

Back-pressure valve

Mechanical relief valve

Pressure gauge and transmitter

Optional flowmeter or flow-verification device

Injection check valve

Stroke and/or speed control

Double-diaphragm leak detection option

Metallic or non-metallic wetted materials

PLC/HMI and remote-system interface

Engineering Approach

The LNS-DZ-PDD design considers:

chemical concentration and compatibility,

minimum and maximum dosing rate,

suction pressure and source-tank level,

process backpressure,

fluid vapor pressure and gas-release tendency,

required accuracy and turndown,

diaphragm and valve material,

pulsation and flowmeter stability,

safe relief routing and calibration procedure.

The selected operating range should avoid excessively low stroke settings where metering accuracy and check-valve performance may deteriorate.

Chemical Dosing Safety Philosophy

Typical permissives and protection may include:

source tank level healthy,

pump available and selected,

injection route established,

discharge pressure within range,

flow verification healthy where provided,

diaphragm leak detector healthy,

relief path available,

ESD chain healthy,

local/remote mode confirmed.

Loss of a critical permissive can stop the affected dosing line and alarm the plant control system.

Advantages

Accurate and repeatable chemical metering

Process liquid separated from drive mechanism

Broad metallic and non-metallic material options

Suitable for corrosive and hazardous chemicals

Calibration and flow-verification capability

Double-diaphragm leak-detection option

Good ratio and batch-control performance

Compact duty/standby arrangement

Typical Applications

Water and wastewater treatment

Boiler and cooling-water treatment

Chemical and petrochemical plants

Refinery chemical dosing

Oil and gas treatment systems

Terminal additive injection

Industrial utility systems

3. LNS-DZ-PDP

Piston / Plunger Dosing Skid System

Product Overview

The LINESIG LNS-DZ-PDP is a high-pressure dosing skid for chemical and hydrocarbon injection where process backpressure is beyond the practical range of centrifugal systems and rugged positive-displacement delivery is required.

The system uses piston or plunger metering pumps with engineered suction conditions, pulsation control, pressure protection, packing or seal management and local or remote capacity control.

Typical Media

Methanol

Corrosion inhibitors

Scale inhibitors

Demulsifiers

Biocides

Solvents

Fuel and refinery additives

Neutralizing agents

Selected catalysts and process chemicals

Key Features

High-pressure piston or plunger metering pump

Single, 1W+1S, 2W+1S, multi-head or independent trains

Manual or electric stroke control

VFD speed control

Combined stroke and speed control option

Suction and discharge pulsation dampeners

Mechanical relief valve

Back-pressure device

Discharge pressure gauge and transmitter

Optional flowmeter or flow-verification device

Packing leakage collection or detection

High-pressure tubing and fittings

PLC/HMI and DCS integration

Engineering Approach

The LNS-DZ-PDP hydraulic design considers:

minimum, normal and maximum injection flow,

normal and maximum process backpressure,

suction acceleration and NPSH margin,

fluid compressibility and gas entrainment,

pulsation dampener sizing,

piping vibration and support,

packing compatibility and leakage management,

relief-valve set pressure and return routing.

High-Pressure Protection Philosophy

Typical protection may include:

mechanical relief valve,

high discharge-pressure alarm and trip,

low suction level or pressure inhibit,

discharge check and injection valve,

packing leak detection,

pump drive fault monitoring,

local emergency stop and remote ESD,

closed drain or drip tray where required.

Advantages

High and very-high-pressure injection capability

Accurate positive-displacement delivery

Rugged continuous-duty construction

Flexible stroke and speed control

Engineered pulsation management

Suitable for hydrocarbon and process chemicals

Integrated pressure protection

Duty/standby and multi-point injection options

Typical Applications

Oil and gas production

Pipeline chemical injection

Refineries

Petrochemical plants

High-pressure process lines

Terminal additive systems

Industrial chemical injection packages

4. LNS-DZ-PDG

Precision Micro Gear Dosing Skid System

Product Overview

The LINESIG LNS-DZ-PDG is a precision positive-displacement micro-gear dosing skid developed for very low-flow, high-repeatability chemical and additive injection.

Closely fitted rotating gears deliver near-continuous, low-pulsation flow and respond accurately to precision speed control.

Typical Media

Fuel dyes and markers

Lubricity improvers

Performance additives

Catalysts

Specialty solvents

Lube-oil additives

Modifiers and inhibitors

Clean, non-abrasive process liquids

Key Features

Precision external or internal micro-gear pump

Near-continuous, low-pulsation delivery

Servo or VFD precision speed control

Single, duty/standby or multi-line package

Fine suction filtration

Optional duplex filter

High-accuracy low-flow meter

Pressure gauge and transmitter

Relief or bypass arrangement

Fine control or injection valve

Mechanical-seal or magnetic-drive option where available

Day tank or small buffer-vessel option

PLC/HMI and remote automation interface

Engineering Approach

The LNS-DZ-PDG design considers:

minimum and maximum dosing rate,

differential pressure and internal slip,

viscosity across the operating-temperature range,

liquid lubricity,

vapor pressure and entrained gas,

maximum particle size and cleanliness,

meter rangeability and speed resolution,

mechanical-seal or magnetic-drive compatibility.

A flooded or positive suction condition is preferred. Suction piping is kept short and low-loss, with filtration selected from the pump manufacturer’s clearance and cleanliness limits.

Filtration and Viscosity Philosophy

Precision micro-gear pumps depend on close internal clearances. Stable performance therefore requires clean liquid, correctly sized suction filtration and controlled viscosity.

Very low viscosity can increase internal slip and reduce volumetric efficiency.

High viscosity can increase suction loss and drive torque.

Heating or heat tracing may be incorporated for viscous additives.

Abrasive solids, crystallizing liquids and polymerizing fluids require special review.

Calibration should cover the actual operating flow and viscosity range.

Advantages

Excellent repeatability at very low flow

Near-continuous, low-pulsation delivery

Fine electronic capacity control

Compact multi-line construction

High-accuracy flow-feedback option

Mechanical-seal or magnetic-drive alternatives

Good fit for dyes, markers, catalysts and specialty additives

Easy integration with ratio and batch control

Typical Applications

Fuel terminals

Refineries and petrochemical plants

Lubricant blending plants

Fine-chemical manufacturing

Catalyst and reagent systems

Pilot and process-development plants

OEM precision-dosing packages

Dye and marker injection systems

Final Selection Basis

Final LNS-DZ model selection requires the chemical identity and concentration, minimum/normal/maximum flow, suction conditions, normal and maximum discharge pressure, density, viscosity, vapor pressure, temperature, solids content, accuracy, turndown, materials, hazardous-area classification and control philosophy.

LINESIG designs and delivers each package against the approved project datasheet, P&ID, operating philosophy and applicable standards.