## Tanker Loading Metering Skids

# LINESIG LNS-MS Series

### Accurate Metering. Safe Loading. Reliable Control.

The LINESIG LNS-MS Series is a modular family of skid-mounted metering systems engineered for accurate, controlled and traceable tanker loading, unloading and custody-transfer applications.

The family is designed around Coriolis mass flow measurement as the preferred metering technology, integrated with explosion-proof batch control, flow regulation, process instrumentation, tanker grounding, overfill protection, ESD interfaces and terminal automation.

Rather than applying one generic metering package to every fluid, the LNS-MS Series uses a service-specific engineering approach for:

- refined liquid hydrocarbons,

- fuel oils and high-viscosity base/lube oils,

- pressurised LPG,

- cryogenic LNG.

This allows the skid configuration, metallurgy, pressure class, flow-control philosophy, heating or cryogenic requirements and safety architecture to be selected according to the actual product and operating conditions.

# LNS-MS PRODUCT FAMILY

| Model | Service | Preferred Metering Technology | Primary Engineering Focus |
| --- | --- | --- | --- |
| LNS-MS-LH | Refined & Light Liquid Hydrocarbons | Coriolis Mass Flowmeter | Batch accuracy, filtration, air elimination, loading safety |
| LNS-MS-LO | Fuel Oil, Base Oil & Lube Oil | Coriolis preferred; PD optional where justified | Viscosity, heating, pressure drop, temperature |
| LNS-MS-LPG | Liquefied Petroleum Gas | Coriolis Mass Flowmeter | Liquid-phase control, pressure, ESD, vapour equalisation |
| LNS-MS-LNG | Liquefied Natural Gas | Cryogenic Coriolis Mass Flowmeter | Cryogenic design, cooldown, BOG, thermal contraction |

# Common LNS-MS Advantages

- Skid-mounted, factory-integrated construction

- Coriolis mass flowmeter based measurement

- Direct mass measurement

- High repeatability

- Explosion-proof batch controller

- Preset quantity loading

- Automatic batch termination

- Flow-control / batch valve

- Pressure monitoring

- Temperature monitoring where required

- Tanker static-grounding interface

- Overfill protection interface

- Emergency shutdown interface

- Loading arm / hose integration

- TAS / PLC / SCADA connectivity

- Transaction and batch data logging

- Modular and customizable design

- Reduced field installation work

- Factory assembly and pre-testing

- Suitable for hazardous-area applications

- Project-specific materials and pressure classes

# Custom Engineering Options

The LNS-MS family can be configured with:

- single or duplex filtration,

- proving/calibration connections,

- additive injection,

- product sampling,

- local flow computer,

- density measurement,

- temperature compensation,

- standard-volume calculations,

- vapour recovery,

- vapour equalisation,

- heating/tracing,

- insulation,

- recirculation,

- emergency release coupling,

- top-loading arms,

- bottom-loading arms,

- loading hoses,

- printer,

- RFID,

- remote I/O,

- skid canopy,

- stainless-steel skid construction,

- custom hazardous-area certification.

# LNS-MS Family Comparison

| Engineering Feature | LNS-MS-LH | LNS-MS-LO | LNS-MS-LPG | LNS-MS-LNG |
| --- | --- | --- | --- | --- |
| Fluid condition | Ambient liquid | Viscous/heated liquid | Pressurised liquid | Cryogenic liquid |
| Preferred meter | Coriolis | Coriolis | Coriolis | Cryogenic Coriolis |
| PD meter option | Normally no | Where justified | Normally no | No |
| Air eliminator | Common | Service-dependent | Not conventional type | Not applicable |
| Heating / tracing | Normally not required | Frequently applicable | Normally not | Cryogenic insulation instead |
| Vapour connection | PMS where required | Service-dependent | Vapour equalisation | BOG/vapour return |
| ESD | Yes | Yes | High importance | High importance |
| Grounding | Yes | Yes | Yes | Project-specific loading interface |
| Overfill interface | Yes | Yes | Yes | Project-specific |
| Cryogenic design | No | No | No | Yes |
| Typical loading | Top / Bottom | Top / Bottom / Side | Pressurised Arm/Hose | Cryogenic Arm/Hose |
| Main design driver | Accuracy / flow conditioning | Viscosity / heat / ΔP | Flash margin / pressure | Temperature / BOG / thermal movement |

# Typical Control:

| # Batch Control | # Static grounding & Overfill protection | # Terminal Automation integration |
| --- | --- | --- |
| - preset batch entry, - live flow indication, - delivered quantity, - target quantity, - batch countdown, - start command, - stop command, - FCV control, - mass totalisation, - product selection, - alarm indication, - transaction recording, - operator identification, - RFID/card interface where required, - digital I/O, - communication with PLC/TAS/SCADA. | - tanker ground verification, - continuous grounding monitoring, - permissive contact to batch controller, - automatic loading inhibit, - loading shutdown on loss of ground, - independent tanker overfill input, - ESD logic, - local status indication, - remote TAS/PLC indication. | Typical interfaces include: - PLC - SCADA - DCS - Terminal Automation System - Batch management - Historian - ERP - Dispatch system - RFID / access control - Ticket printer - Vehicle identification Typical communication options may include: - Modbus RTU - Modbus TCP - Industrial Ethernet - Pulse - Frequency - Analog I/O # Digital I/O |

# Typical LNS-MS Process Architecture

Product Inlet → Isolation Valve → Strainer / Filter → Air/Gas Management where applicable → Pressure / Temperature Measurement → Coriolis Mass Flowmeter → Flow / Batch Control Valve → Isolation / Check Valve → Loading Arm or Hose → Tanker

The exact process sequence is modified according to service.

For example:

- atmospheric liquid hydrocarbons may use an air eliminator;

- high-viscosity oils may require heating and tracing;

- LPG may require vapour-return/equalisation;

- LNG requires cryogenic equipment, cooldown and BOG management.

- Model Selection

- Choose:

- LNS-MS-LH
for gasoline, diesel, kerosene, Jet A1 and similar refined liquid hydrocarbons.

- LNS-MS-LO
for fuel oil, base oil, lube oil and other viscous or temperature-sensitive oil products.

- LNS-MS-LPG
for pressurised propane, butane and LPG loading.

- LNS-MS-LNG
for cryogenic LNG loading and transfer.

# 1. LNS-MS-LH

## Liquid Hydrocarbon Tanker Loading Metering Skid

### Product Overview

The LINESIG LNS-MS-LH is a skid-mounted tanker loading metering system developed for refined and light liquid hydrocarbon products.

It is designed for controlled and traceable transfer from storage tanks, transfer headers or loading pumps to road tankers, railcars or marine-loading systems.

The skid combines flow conditioning, Coriolis mass measurement, batch control, flow regulation and loading-safety interfaces in a compact package.

### Typical Products

- Gasoline / PMS

- Diesel / AGO

- Kerosene

- Jet A1

- Naphtha

- Light petroleum products

- Similar refined hydrocarbons

## Key Features

- Coriolis mass flowmeter preferred

- Product-specific strainer/filter

- Air eliminator where required

- Pressure gauge

- Pressure transmitter

- Temperature transmitter where required

- Flow / batch control valve

- Explosion-proof batch controller

- Static-grounding interface

- Overfill protection interface

- ESD pushbutton / shutdown interface

- Loading-arm or hose connection

- PMS vapour-recovery interface where required

- Dedicated clean-service configuration available for Jet A1

## Engineering Approach

The LNS-MS-LH design focuses on maintaining:

- clean product flow,

- stable single-phase liquid conditions,

- accurate batch measurement,

- controlled loading rate,

- tanker safety,

- reliable transaction records.

For Jet A1 service, filtration, cleanliness, water management, sampling and material requirements are reviewed separately according to the project specification.

For PMS/gasoline applications, the skid can interface with the terminal vapour-return or vapour-recovery system.

## Advantages

- Direct mass measurement

- Improved loading accuracy

- Reduced dependence on separate density compensation

- Reliable batch preset and termination

- Compact loading-bay installation

- Reduced field piping and cabling

- Suitable for multi-product terminals

- Supports automatic tanker-loading operations

- Easy integration with TAS and terminal dispatch systems

## Typical Applications

- Fuel depots

- Petroleum terminals

- Refineries

- Tank farms

- Airport fuel terminals

- Marine terminals

- Road tanker loading gantries

- Rail loading systems

- Distribution terminals

# 2. LNS-MS-LO

## Fuel Oil / Base Oil / Lube Oil Metering Skid

### Product Overview

The LINESIG LNS-MS-LO is specially engineered for tanker loading of medium- to high-viscosity liquid products such as fuel oils, base oils and finished lubricating oils.

Unlike conventional light-hydrocarbon metering, these applications require careful consideration of viscosity, operating temperature, pressure drop and heating requirements.

Coriolis mass flow measurement remains the preferred LINESIG approach wherever technically suitable.

## Typical Products

- Fuel Oil

- Heavy Fuel Oil

- Furnace Oil

- Base Oil SN150

- Base Oil SN500

- Bright Stock

- Lube Base Stocks

- Finished Lubricants

- Industrial Oils

- High-viscosity petroleum products

## Key Features

- Coriolis mass flowmeter preferred

- Positive-displacement meter option where justified

- Low-pressure-drop strainer/filter

- Temperature measurement

- Pressure measurement

- Flow / batch control valve

- Explosion-proof batch controller

- Heating/tracing options

- Thermal insulation options

- Recirculation / warm-up interface

- Static grounding

- Overfill protection

- ESD interface

- TAS/PLC integration

## Engineering Approach

For high-viscosity products, the skid is sized based not only on normal operating conditions but also on the highest credible viscosity at the lowest operating temperature.

The design may therefore include:

- steam tracing,

- thermal-oil tracing,

- electric tracing,

- insulation,

- heated filters,

- jacketed equipment,

- recirculation,

- warm-up lines.

The Coriolis meter and control valve are selected together with the system hydraulic calculations to ensure acceptable pressure drop.

Where Coriolis pressure drop becomes excessive because of viscosity, line size or very high capacity, a positive-displacement meter may be evaluated as an alternative.

## Advantages

- Suitable for high-viscosity services

- Accurate mass measurement across multiple oil grades

- Reduced loading-time variation

- Better control at changing product temperatures

- Integrated temperature monitoring

- Supports heated loading systems

- Flexible for base-oil and lubricant terminals

- Automated batch loading

- Reduced operator dependency

## Typical Applications

- Base-oil terminals

- Lubricant blending plants

- Fuel-oil terminals

- Heavy-oil storage facilities

- Refineries

- Marine fuel terminals

- Industrial oil depots

- Tanker loading stations

# 3. LNS-MS-LPG

## LPG Tanker Loading Metering Skid

### Product Overview

The LINESIG LNS-MS-LPG is a pressurised-liquid metering skid designed for safe and accurate loading of liquefied petroleum gas.

The system integrates Coriolis mass metering, pressure and temperature monitoring, explosion-proof batch control, ESD functions, tanker grounding, overfill protection and vapour-return/equalisation interfaces.

LPG metering requires a dedicated hydraulic design because the product must remain in the liquid phase throughout the metering system.

## Typical Products

- Propane

- Butane

- Propane/Butane mixtures

- Commercial LPG

- Pressurised liquefied hydrocarbon gases

## Key Features

- Coriolis mass flowmeter

- Pressure-rated skid piping

- LPG-compatible materials

- Pressure gauge

- Pressure transmitter

- Temperature transmitter

- Fail-safe flow / batch / ESD valve

- Explosion-proof batch controller

- Static-grounding interface

- Overfill protection

- Vapour-return / equalisation connection

- Loading-arm / hose interface

- Breakaway/ERC option

- TAS / PLC / SCADA interface

## Engineering Approach

The skid hydraulic design ensures adequate pressure is maintained through:

- inlet piping,

- strainer,

- meter,

- control valve,

- loading arm,

- tanker connection.

This reduces the risk of flashing and two-phase flow through the Coriolis meter.

The system can also incorporate a separate vapour-return or pressure-equalisation connection between the tanker and terminal vapour system.

## LPG Safety Philosophy

Typical permissives may include:

- tanker grounding healthy,

- overfill system healthy,

- correct product route established,

- meter healthy,

- batch controller healthy,

- ESD chain healthy,

- loading arm connected,

- vapour return ready,

- pressure within acceptable range.

Loss of a critical permissive can initiate controlled or emergency loading shutdown.

## Advantages

- Direct LPG mass measurement

- High loading accuracy

- Reduced dependence on volumetric correction

- Automated preset loading

- Improved tanker loading safety

- Integrated vapour equalisation

- ESD-ready architecture

- Compact skid design

- Suitable for custody-transfer applications where configured accordingly

## Typical Applications

- LPG terminals

- LPG filling plants

- Refinery LPG dispatch

- Bulk LPG storage facilities

- LPG road tanker loading

- Rail LPG loading

- Marine LPG loading

- LPG distribution depots

# 4. LNS-MS-LNG

## LNG Cryogenic Tanker Loading Metering Skid

### Product Overview

The LINESIG LNS-MS-LNG is a dedicated cryogenic tanker-loading metering system for liquefied natural gas.

The skid is designed around cryogenic Coriolis mass measurement together with LNG-compatible piping, valves, instrumentation, insulation, ESD interfaces and boil-off/vapour management provisions.

The LNS-MS-LNG is not simply a modified ambient-temperature skid. It is engineered specifically for cryogenic service.

## Typical Product

- Liquefied Natural Gas

- Cryogenic methane-rich LNG mixtures

- Project-specific LNG compositions

## Key Features

- Cryogenic Coriolis mass flowmeter

- Cryogenic stainless-steel process piping

- Cryogenic isolation valves

- Cryogenic FCV / batch / ESD valves

- Pressure transmitters

- Cryogenic temperature transmitters

- Explosion-proof batch control

- BOG/vapour-return connection

- Loading-arm / hose/ERC interface

- Gas-detection interface

- ESD system interface

- Insulated or vacuum-jacketed piping option

- Thermal-contraction accommodation

## Engineering Approach

The LNS-MS-LNG design considers:

- cryogenic operating temperature,

- cooldown procedure,

- thermal contraction,

- pipe flexibility,

- support loads,

- insulation,

- external heat ingress,

- boil-off gas generation,

- pressure balance,

- cryogenic valve design,

- emergency release.

The metering skid is integrated into the wider LNG loading sequence rather than treated as a standalone meter spool.

## Typical LNG Loading Sequence

System Ready → Cryogenic Cooldown → Stable Liquid Conditions → Batch Start → Cryogenic Coriolis Metering → Controlled Transfer → Batch Completion → Valve Closure → Safe Isolation

Where required, vapour/BOG return operates simultaneously.

## Advantages

- Direct LNG mass measurement

- Cryogenic-specific engineering

- Integrated cooldown monitoring

- Compact packaged construction

- Reduced field assembly

- ESD and ERC integration

- BOG/vapour-return compatibility

- Suitable for terminal automation

- High level of operational control

## Typical Applications

- LNG truck loading

- LNG truck unloading

- LNG satellite stations

- LNG terminals

- LNG bunkering facilities

- Cryogenic transfer stations

- Marine LNG loading

- Small-scale LNG distribution systems