# LINESIG SCRUBBER SYSTEMS

## Cleaner Industry. Healthier Air. Greener Tomorrow.

LINESIG Scrubber Systems are engineered for the treatment of contaminated industrial air, process gases, odorous emissions, acid gases, alkaline vapours, particulate matter and sulfur-bearing flue gases.

Our scrubber range combines mechanical separation, gas-liquid absorption, chemical neutralization, biological degradation and dry sorbent technologies to match different process conditions, contaminant types and environmental requirements.

The product family includes:

LNS-SC-BIO — Biological Scrubber
LNS-SC-WET — Wet Packed-Bed Scrubber
LNS-SC-DRY — Dry Scrubber
LNS-SC-VEN — Venturi Scrubber
LNS-SC-FGD — Flue Gas Desulfurization System

# Product Range

## LNS-SC-BIO

### Industrial Bio-Scrubber System

The LNS-SC-BIO is designed for continuous treatment of biodegradable, water-soluble VOCs and odorous industrial gases.

Contaminated gas passes through a wetted packed section where pollutants are absorbed into the recirculating liquid and subsequently degraded by acclimated microorganisms.

### Best suited for

- Hydrogen Sulfide — H₂S

- Ammonia — NH₃

- Mercaptans

- Amines

- Volatile Fatty Acids

- Water-soluble VOCs

- Organic odours

- Wastewater and biological-process exhaust

### Typical Industries

Wastewater treatment, food processing, rendering, fermentation, pulp & paper, composting, biogas and selected chemical/pharmaceutical processes.

### Key Advantages

- Low chemical consumption

- Sustainable biological treatment

- High odour-removal capability

- Continuous operation

- Corrosion-resistant construction

- Low secondary waste generation

- Integrated pH/nutrient control

Technology: Biological absorption + biodegradation

# LNS-SC-WET

### Industrial Wet Packed-Bed Scrubber

The LNS-SC-WET is a counter-current gas-liquid absorption system designed for chemically reactive and water-soluble contaminants.

Gas flows upward through a packed bed while recirculated scrubbing liquid flows downward, allowing contaminants to dissolve and react with the selected chemical reagent.

### Best suited for

- HCl

- HF

- SO₂

- NH₃

- H₂S

- Chlorine

- Acid fumes

- Alkaline vapours

- Mercaptans

- Soluble VOCs

- Industrial odours

### Typical Industries

Chemical, petrochemical, fertilizer, metal pickling, galvanizing, wastewater treatment, textile, pulp & paper, food processing and chemical storage facilities.

### Key Advantages

- High gas removal efficiency

- Flexible chemical treatment

- Compact vertical footprint

- Automated pH/ORP control

- High-efficiency mist elimination

- Multiple corrosion-resistant MOC options

- Single or multi-stage design

Technology: Physical absorption + chemical neutralization / oxidation

# LNS-SC-DRY

### Dry Sorbent Scrubber System

The LNS-SC-DRY removes gaseous pollutants without a continuously circulating liquid system.

A dry or semi-dry reagent is introduced into the contaminated gas stream, where pollutants react with the sorbent and form stable solid reaction products that are subsequently removed by filtration or dust-collection equipment.

### Best suited for

- SO₂

- HCl

- HF

- Acid gases

- Selected sulfur compounds

- Incineration gases

- Combustion exhaust

- Processes where wastewater generation should be minimized

### Typical Industries

Waste-to-energy, incineration, cement, lime, metals, chemical plants, furnaces, boilers and industrial combustion systems.

### Typical Reagents

- Hydrated lime — Ca(OH)₂

- Sodium bicarbonate — NaHCO₃

- Lime-based sorbents

- Application-specific powdered reagents

### Key Advantages

- No continuous liquid circulation

- Low wastewater generation

- Compact process arrangement

- Suitable for high-temperature gas treatment

- Simple chemical-storage and injection concept

- Compatible with bag filters and dust collectors

Technology: Dry reagent injection + gas-solid reaction

# LNS-SC-VEN

### Venturi Scrubber System

The LNS-SC-VEN is a high-energy wet scrubbing system designed primarily for fine particulate, mist and aerosol removal.

Contaminated gas is accelerated through a Venturi throat where scrubbing liquid is introduced. High relative velocity between gas and liquid creates very fine droplets that capture suspended particles and soluble contaminants.

### Best suited for

- Fine particulate matter

- Dust

- Acid mist

- Oil mist

- Aerosols

- Smoke

- Soluble gases combined with particulate loading

- Sticky or difficult dust streams

### Typical Industries

Metallurgy, foundries, mineral processing, fertilizer, chemical plants, dryers, furnaces, incineration, battery plants and process exhaust systems.

### Key Advantages

- High particulate capture efficiency

- Effective on sub-micron droplets and aerosols

- Handles high dust loading

- Simultaneous gas and particulate treatment

- Resistant to sticky particulate service

- Can be combined with packed scrubbers

Technology: High-energy gas-liquid impaction

# LNS-SC-FGD

### Flue Gas Desulfurization System

The LNS-SC-FGD is engineered for sulfur-oxide removal from industrial combustion and process flue gases.

Depending on project requirements, the system can use wet limestone, lime, caustic, seawater, dry sorbent or semi-dry treatment technology.

### Best suited for

- Sulfur Dioxide — SO₂

- Sulfur Trioxide — SO₃

- Acid gases

- Combustion-generated sulfur emissions

- Boiler and furnace flue gas

- High-volume industrial exhaust

### Typical Industries

Power generation, boilers, refineries, petrochemical heaters, cement plants, metal processing, waste incineration, marine applications and large industrial furnaces.

### Typical FGD Technologies

- Wet limestone FGD

- Lime scrubbing

- Caustic scrubbing

- Seawater FGD

- Dry sorbent injection

- Semi-dry / spray dryer absorber

### Key Advantages

- Suitable for very large gas volumes

- High SO₂ removal capability

- Multiple reagent technologies

- Integrated oxidation and blowdown options

- Gypsum recovery possible for limestone systems

- Fully automated reagent and emission control

Technology: Sulfur absorption / neutralization

# Quick Selection Guide

| Contaminant / Duty | BIO | WET | DRY | VEN | FGD |
| --- | --- | --- | --- | --- | --- |
| H₂S / odour | Excellent | Excellent | Good | Moderate | Limited |
| NH₃ | Excellent | Excellent | Moderate | Limited | Limited |
| HCl / HF | Limited | Excellent | Excellent | Good | Good |
| SO₂ | Limited | Excellent | Excellent | Moderate | Best |
| Mercaptans | Excellent | Excellent | Moderate | Limited | Limited |
| Water-soluble VOCs | Excellent | Excellent | Limited | Limited | Limited |
| Acid mist | Limited | Good | Limited | Excellent | Moderate |
| Fine particulate | Limited | Moderate | With filter | Excellent | Moderate |
| High dust loading | No | Limited | Good | Excellent | Moderate |
| Large boiler/flue-gas duty | No | Possible | Good | Pre-cleaning | Best |
| Minimum wastewater | Good | Moderate | Best | Moderate | Depends on FGD type |
| Biological treatment | Yes | No | No | No | No |
| Chemical reagent use | Low/limited | Yes | Yes | Optional | Yes |
| Best for variable corrosive gas | Moderate | Excellent | Good | Good | Good |
| High-temperature gas | Limited | Requires cooling | Good | Good with quench | Excellent |

# Selection by Process Requirement

Choose LNS-SC-BIO when:
the main problem is biodegradable odour, H₂S, ammonia or soluble organic compounds and low chemical consumption is preferred.

Choose LNS-SC-WET when:
the gas contains corrosive or reactive contaminants requiring controlled chemical neutralization, oxidation or absorption.

Choose LNS-SC-DRY when:
wastewater generation should be minimized and the application involves acid gases from combustion or thermal processes.

Choose LNS-SC-VEN when:
the main contaminant is fine particulate, mist, aerosol, smoke or a combined gas/particle stream.

Choose LNS-SC-FGD when:
the application is primarily sulfur-oxide control from boilers, furnaces, power plants or other large flue-gas sources.

# Combined Treatment Systems

For difficult industrial emissions, LINESIG scrubbers can be configured as multi-stage treatment systems.

Typical combinations include:

- Venturi + Wet Packed Bed for dust, mist and soluble gas

- Wet Scrubber + Mist Eliminator + Carbon Polishing

- Bio-Scrubber + Chemical Polishing Stage

- FGD + Particulate Pre-Cleaning

- Dry Scrubber + Bag Filter

- Two-Stage Acid / Alkali Wet Scrubber

This allows each pollutant group to be treated in the most effective stage rather than forcing one technology to perform every duty.

# Common Engineering Features

LINESIG scrubber packages can incorporate corrosion-resistant FRP, PP, PVC, PVDF or stainless-steel construction; packed-media sections; internal liquid distributors; mist eliminators; circulation pumps; fresh-water make-up; reagent dosing; pH, ORP and conductivity control; automatic blowdown; level control; differential-pressure monitoring; process gas blowers; local PLC/HMI panels; remote SCADA/DCS communication; access manholes; skid-mounted construction; and duty/standby equipment arrangements.

# Design Inputs

Final scrubber selection and sizing should be based on:

- minimum, normal and maximum gas flow

- gas temperature and pressure

- contaminant composition and concentration

- particulate/aerosol loading

- inlet humidity

- required outlet emission limit

- required removal efficiency

- chemical compatibility

- available water quality

- reagent availability

- wastewater-disposal limits

- hazardous-area classification

- allowable pressure drop

- site ambient and installation conditions

## LINESIG SCRUBBER SYSTEMS

Control Emissions. Protect People. Preserve the Environment.

BIO | WET | DRY | VENTURI | FLUE GAS DESULFURIZATION