# Industrial Scrubber Systems

Cleaner Industry. Healthier Air. Greener Tomorrow.

LINESIG industrial scrubber systems are engineered for the treatment of contaminated process air, odorous gases, chemical vapours and selected industrial emissions. The range combines compact vertical construction, efficient gas–liquid contact, corrosion-resistant materials, integrated recirculation systems and automated process control.

### Product Range

LNS-SC-BIO — Industrial Bio-Scrubber System
Biological treatment system for water-soluble VOCs, hydrogen sulfide, ammonia, mercaptans, amines and complex industrial odours.

LNS-SC-WET — Industrial Wet Scrubber System
Packed-bed chemical wet scrubber for acidic gases, alkaline vapours, hydrogen sulfide, chemical fumes, soluble VOCs, odours and selected aerosols.

# LNS-SC-BIO

## Industrial Bio-Scrubber System

### Cleaner Air Through Biological Treatment

The LINESIG LNS-SC-BIO is a vertical packed-bed biological gas-treatment system engineered for continuous removal of water-soluble volatile organic compounds and odorous industrial emissions.

Contaminated gas passes counter-current through a continuously wetted packed section where pollutants are transferred into the aqueous phase. Specialized microorganisms subsequently metabolize the absorbed contaminants into environmentally benign end products such as carbon dioxide, water and mineral salts.

### Key Benefits

- High-efficiency odour and soluble VOC control

- Biological treatment with reduced chemical consumption

- Suitable for continuous 24/7 industrial operation

- Corrosion-resistant vertical packed-column construction

- Compact footprint

- Integrated water recirculation

- Nutrient and pH dosing capability

- Automated level, pH and recirculation control

- High-efficiency mist eliminator

- Low secondary waste generation

- PLC-based automatic operation

- Environmentally sustainable treatment philosophy

### Typical Contaminants

Hydrogen Sulfide — H₂S
Ammonia — NH₃
Mercaptans
Dimethyl Sulfide
Trimethylamine and other amines
Volatile Fatty Acids
Methanol
Ethanol
Acetone
Formaldehyde
Water-soluble VOCs
Complex organic odours

The BIO datasheet particularly identifies wastewater, chemical/pharmaceutical, food/rendering/fermentation and pulp-and-paper applications.

### Industries

Municipal & Industrial Wastewater Treatment
Headworks, clarifiers, sludge tanks, thickeners, dewatering buildings and sewer ventilation.

Chemical & Pharmaceutical
Process vents, reactor exhaust, solvent handling and water-soluble organic vapours.

Food Processing & Rendering
Cookers, dryers, waste handling, fermentation and odorous process areas.

Pulp & Paper
Reduced sulfur compounds, mercaptans and non-condensable gases.

Biogas & Organic Waste Facilities
H₂S and odorous reduced-sulfur treatment.

Composting & Waste Processing
Ammonia, sulfur compounds and mixed biological odours.

### How It Works

1. Contaminated Gas Entry
Process gas enters the lower portion of the scrubber.

2. Counter-Current Absorption
Gas flows upward through high-surface-area biological packing while recirculating liquid flows downward.

3. Mass Transfer
Soluble contaminants transfer from the gas phase into the liquid film.

4. Biological Oxidation
Acclimated microorganisms metabolize the absorbed contaminants.

5. Mist Removal
A high-efficiency mist eliminator removes entrained liquid droplets.

6. Clean Gas Discharge
Treated gas exits through the upper stack connection.

### Main System Components

- Vertical scrubber column

- Biological packed media

- Packing support grid

- Internal spray distribution system

- Mist eliminator

- Recirculation sump

- Scrubbing-water circulation pump

- Fresh-water make-up

- Water level indicator/controller

- Nutrient dosing system

- Optional pH correction system

- Drain/blowdown connection

- Process gas blower where required

- Electrical control panel

- PLC/HMI automation

- Process instrumentation

### Typical Technical Features

| Parameter | Typical Design Basis |
| --- | --- |
| Configuration | Vertical counter-current packed scrubber |
| Packing | High-surface-area PP biological media |
| Liquid circulation | Continuous |
| Process control | PLC based |
| pH control | Automatic / optional |
| Mist elimination | High-efficiency PP demister |
| Shell MOC | FRP/Vinyl Ester or project-specific |
| Biological monitoring | pH, DO and process-specific parameters |
| Installation | Outdoor / industrial service |
| Capacity | Engineered according to project gas flow |

The existing BIO design basis includes high-surface-area PP packing and a PP chevron-type mist eliminator.

### Available Options

Nutrient dosing, automatic pH adjustment, dissolved-oxygen monitoring, redundant pumps, process gas blower, gas analyzers, conductivity measurement, automated blowdown, remote SCADA connectivity and multi-stage treatment.

### Recommended Website Tagline

Natural Treatment. Cleaner Air. Sustainable Industry.

# LNS-SC-WET

## Industrial Wet Scrubber System

### Engineered Chemical Gas Cleaning

The LINESIG LNS-SC-WET is a vertical packed-bed wet scrubber engineered for the removal of corrosive gases, chemical vapours, industrial odours and water-soluble contaminants from process exhaust streams.

The system combines high-efficiency gas–liquid mass transfer with controlled chemical absorption, neutralization or oxidation. Scrubbing liquid is continuously recirculated through internal spray headers and packed media while the required chemical condition is maintained automatically.

### Key Benefits

- High gas-removal efficiency

- Compact vertical packed-bed design

- Suitable for corrosive industrial gases

- Multiple chemical-treatment modes

- Low pressure-drop packing

- Uniform internal spray distribution

- Integrated recirculation sump

- Automatic fresh-water make-up

- pH / ORP / conductivity control

- Automatic chemical dosing

- Controlled blowdown

- High-efficiency mist elimination

- Corrosion-resistant construction

- PLC/HMI automation

- Single-stage or multi-stage configuration

### Typical Contaminants

Hydrogen Chloride — HCl
Hydrogen Fluoride — HF
Sulfur Dioxide — SO₂
Hydrogen Sulfide — H₂S
Ammonia — NH₃
Chlorine — Cl₂
Mercaptans
Amines
Acid fumes
Alkaline vapours
Water-soluble VOCs
Industrial odours
Selected acid mists and soluble aerosols

### Typical Chemical Operations

| Pollutant | Typical Scrubbing Chemistry |
| --- | --- |
| HCl / HF / SO₂ | NaOH or alkaline solution |
| NH₃ | Dilute acid scrubbing |
| H₂S | NaOH + oxidizing agent |
| Mercaptans | Alkaline / oxidation chemistry |
| Chlorine | Alkaline or reducing treatment |
| Acid mist | Water / alkaline solution |
| Soluble VOCs | Water or application-specific liquor |
| Mixed odour gases | Selected neutralization / oxidation chemistry |

Final reagent selection should always be based on actual gas composition, inlet concentration and discharge requirement.

### Industries

Chemical & Petrochemical
Reactor vents, process tanks, storage vents and chemical handling systems.

Fertilizer & Ammonia Plants
NH₃ and alkaline gas treatment.

Metal Pickling & Galvanizing
HCl, H₂SO₄ and acid-fume removal.

Wastewater Treatment
H₂S, ammonia and odour control.

Pulp & Paper
Sulfur compounds and chemical vapours.

Textile & Dyeing
Acid and alkaline process fumes.

Food & Rendering
Odours, amines and soluble process vapours.

Battery / Recycling Facilities
Acid mist and corrosive emissions.

Bulk Chemical Storage
Tank breathing and transfer vapour treatment.

### How It Works

1. Gas Entry
Contaminated air enters the lower scrubber section.

2. Gas Distribution
The inlet arrangement distributes the gas across the column.

3. Packed-Bed Absorption
Gas travels upward through wetted packing.

4. Chemical Reaction
Contaminants dissolve into the circulating liquor and react with the selected reagent.

5. Spray Distribution
Internal full-cone nozzles continuously wet the packed section.

6. Mist Elimination
Entrained droplets are removed before discharge.

7. Clean Gas Outlet
Treated gas exits to the stack.

8. Liquid Recirculation
The sump liquid is pumped continuously back to the internal spray system.

9. Chemical Control
pH, ORP or conductivity signals regulate chemical dosing and blowdown.

### Main System Components

- Vertical packed scrubber column

- PP/PVDF structured or random packing

- Packing support grid

- Internal spray header

- Full-cone spray nozzles

- Mist eliminator

- Integral recirculation sump

- Circulation pump

- Fresh-water make-up connection

- Automatic make-up valve

- Level indicator/controller

- Chemical dosing tank(s)

- Chemical metering pump(s)

- pH / ORP / conductivity analyzer

- Blowdown/drain system

- Access manhole

- Process gas inlet/outlet

- Electrical control panel

- PLC/HMI system

- Process gas blower where required

### Typical Construction Options

| Component | Available MOC |
| --- | --- |
| Scrubber shell | FRP/Vinyl Ester, PP, PVC, PVDF, SS316L |
| Packing | PP / PVDF |
| Spray piping | PP / PVC / PVDF |
| Spray nozzles | PP / PVDF / SS |
| Mist eliminator | PP / PVDF |
| Pumps | PP / PVDF / SS / FRP-lined |
| Piping | PP / PVC / PVDF / FRP / SS |
| Skid | Coated CS / SS |
| Control panel | Painted steel / SS304 / SS316 |

### Available Configurations

Water Scrubber — for highly water-soluble contaminants.

Alkaline Scrubber — for acidic gases.

Acid Scrubber — for ammonia and alkaline vapours.

Oxidizing Scrubber — for H₂S, mercaptans and selected odours.

Two-Stage Scrubber — when different contaminants require separate chemical reactions.

Venturi + Packed Bed — where particulate or aerosol removal is required before gas absorption.

### Control & Automation

The LNS-SC-WET can be supplied with:

- PLC/HMI control

- Automatic sump-level control

- Fresh-water solenoid/control valve

- Recirculation flow monitoring

- pH control

- ORP control

- Conductivity/TDS monitoring

- Automatic chemical dosing

- Automatic blowdown

- Packed-bed differential pressure

- Blower VFD control

- Pump duty/standby changeover

- Local/remote operation

- Alarm history

- SCADA/DCS communication

### Recommended Website Tagline

Control Emissions. Protect People. Preserve the Environment.